HAND AND POWER TOOLS AND EQUIPMENT INSPETION

Department/Division:		 Date Of Inspection: 		 Location: 	 Inspector:

I Criteria	I Yes I No I NIA I

HAND TOOLS AND EQUIPMENT

| Are tools and equipment (City and personal) in good condition? | □ | □ | □ |
| --- | --- | --- | --- |
| Are chisels, punches or other mushroomed head tools repaired or replaced? | □ | □ | □ |
| Are broken handles on hammers and axes replaced promptly? | □ | □ | □ |
| Are worn or bent wrenches repaired or replaced? | □ | □ | □ |
| Do files have handles? | □ | □ | □ |
| Is eye and face protection worn while using hand tools that might produce flying materials or breakage? | □ | □ | □ |
| Have·employees been trained to use hand tools properly? | □ | □ | □ |
| Are jacks checked to assure they are in good operating condition and marked with the Jack capacity? | □ | □ | □ |

PORTABLE POWER TOOLS AND EQUIPMENT

| Are grinders, saws and similar equipment used with appropriate safety guards? | □ | □ | □ |
| --- | --- | --- | --- |
| Are portable circular saws equipped with guards above and below the base shoe? | □ | □ | □ |
| Are rotating or moving parts guarded to prevent physical contact? | □ | □ | □ |
| Are all cord-connected, electrically operated tools and equipment grounded or double insulated? | □ | □ | □ |
| Are guards in placed over belts, pulleys, chains and sprockets on equipment such as concrete mixers, air compressors, etc.? | □ | □ | □ |
| Are portable fans provided with full guards having openings of½ inch or less? | □ | □ | □ |
| Are Ground Fault Circuit Interrupters (GFCI) used with portable electrical power tools? | □ | □ | □ |
| Is compressed air used for cleaning reduced to a nozzle pressure of 30 psi or less? | □ | □ | □ |
| Are pneumatic and hydraulic hoses on power-operated tools inspected regularly for serviceability? | □ | □ | □ |
| Is portable hoisting equipment posted with capacity and latest load test information? | □ | □ | □ |
| Do chain saws have anti-kickback devices? | □ | □ | □ |

ABRASIVE WHEEL GRINDERS

| Is the work rest adjusted to within 1/8 inch on the wheel? | □ | □ | □ |
| --- | --- | --- | --- |
| Is the tongue guard adjusted to within ¼ inch of the wheel? | □ | □ | □ |
| Do side guards cover the spindle, nut and flange and 75% of the wheel diameter? | □ | □ | □ |
| Are bench and pedestal grinders permanently mounted? | □ | □ | □ |
| Are goggles or face shields always worn while grinding? | □ | □ | □ |
| Is the maximum RPM rating of each abrasive wheel compatible with the RPM rating of the grinder motor? | □ | □ | □ |
| Does each grinder have an individual on and off control? | □ | □ | □ |
| Are dust collectors or powered exhausts provided? | □ | □ | □ |

POWER ACTUATED TOOLS

| Are employees who operate power-actuated tools trained in their use and do they carry a valid operators card? | □ | □ | □ |
| --- | --- | --- | --- |
| Is each power-actuated tool stored in its own locked container when not being used? | □ | □ | □ |

| Is a sign at least 7" x 1O" with bold face type reading "POWER ACTUATED TOOL IN USE" conspicuously placed to warn others that the tool is beinq used? | □ | □ | □ |
| --- | --- | --- | --- |
| Are power-actuated tools left unloaded until they are ready to be used? | □ | □ | □ |
| Are power actuated tools inspected for obstructions or defects each day before use? | □ | □ | □ |
| Do power actuated tool operators have and use appropriate personal protective equipment (head, eye, hearinq, etc.)? | □ | □ | □ |
| ADDITIONAL REMARKS: |  |  |  |